# Phoenix MTU-5A to MTH5

User Manual — legacy TSn conversion with raw ADC-count preservation

Based on the supplied Python source code • October 2026

# 1. Purpose and processing sequence

This tool reads legacy Phoenix MTU-5A TSn files and exports a multi-run MTH5 while preserving the numerical time series as original ADC counts.

Recommended workflow: legacy TSn + TBL → raw-count MTH5; original Phoenix SysCal + TBL/CLB/CLC → CTS; then MTH5 + CTS → pyTRACMT processing.

# 2. Installation and launch

Install Python and numpy, pandas, matplotlib, mth5, and mt_metadata in a compatible environment.

Example: python -m pip install numpy pandas matplotlib mth5 mt_metadata

GUI: python "Phoenix_MTU5A_to_MTH5(6).py"

Folder CLI: python "Phoenix_MTU5A_to_MTH5(6).py" --folder "PATH_TO_SURVEY"

# 3. Input data and acquisition parameters

The folder scanner searches for .TBL, .TS2/.TS3/.TS4/.TS5 (and supported additional TSn extensions), .CLB, .CLC, and .CLE, including shallow calibration subfolders.

TBL provides recording and channel settings such as sample rate, electrode length, and gain codes. The program matches TSn and TBL by recording filename/family when possible.

Nominal fallback sample rates are TS2=24000 Hz, TS3=2400 Hz, TS4=150 Hz, TS5=15 Hz; verify actual rates against TBL.

# 4. GUI operating procedure

1. Click “Select folder”, then “Scan folder and load”. Inspect the discovered TBL, TSn, and calibration files.

2. Alternatively choose “Select TBL(s)” and “Select TSn files” manually.

3. Review acquisition settings, electrode lengths, and gain values; correct erroneous entries where supported by the GUI.

4. Use “Select CLB”, “Select CLC/CLE(s)”, “Auto Detect Coil Calibration”, and “Show Match Report” to inspect calibration associations.

5. Use “Load selected TS data” and plotting controls to inspect raw recordings and continuity.

6. Choose “Auto convert folder → MTH5” or “Export MTH5 (Auto Multi-Run)” and specify output.

7. Optional diagnostics include CSV, merged absolute-time CSV, JSON/LOG, and calibration plots.

# 5. Raw-count and calibration policy

The MTH5 waveform remains in counts: it is NOT preconverted to mV/km, nT, or volts. This avoids ambiguous gain and calibration being applied twice.

The GUI may display estimated input voltage for inspection, using ADC_volts = counts / 2^24 × full_scale_range_Vpp and input_volts = ADC_volts / gain. Display conversion does not change stored raw-count data.

The program can export EMpower-style metadata and calibration JSON and mount FAP/ADC filters with applied=False. Its proprietary CLB/CLC/CLE decoding is heuristic and requires independent validation.

The separate manufacturer-produced CTS file should be used as the authoritative calibration input in the intended pyTRACMT workflow. Do not assume a reconstructed JSON response is identical to CTS.

# 6. Export structure and quality checks

Exported MTH5 organizes separate sample-rate segments/runs and preserves the original counts. Verify run names, timestamps, sample counts, channel assignment, and electrode geometry.

Where recordings overlap, check the time-continuity and merging reports before accepting a combined time series.

# 7. Using the output with pyTRACMT

Load the exported raw-count MTH5, select the appropriate run and CTS response, and confirm pyTRACMT’s count-to-physical-unit and response-calibration settings.

Inspect resulting apparent resistivity and phase across TS frequency bands, especially at band transitions and where calibrations overlap.

# 8. Troubleshooting

Missing TBL or uncertain pairing: select the matching file manually. Incorrect electrode length: correct the acquisition geometry before processing.

Missing coil response: verify serial identifiers and calibration directory. Implausible amplitude: check ADC full-scale and gain assumptions, and whether calibration was inadvertently applied twice.