# Phoenix SysCal GUI v3.1

User Manual — graphical TBL editor and manufacturer SysCal CTS generation

Based on the supplied Python source code • October 2026

# 1. Purpose

Phoenix SysCal GUI v3.1 is a Windows graphical front end for the manufacturer’s SysCal-V6.exe. It does not implement an independent CTS calibration algorithm: the manufacturer executable performs calibration.

The GUI helps locate inputs, inspect and edit binary TBL parameters, run SysCal, and collect a CTS output for later pyTRACMT processing.

# 2. Requirements and launch

Windows and a valid manufacturer-supplied SysCal-V6.exe are required for actual CTS generation. The GUI itself uses Python standard-library modules including tkinter.

Launch: python Phoenix_SysCal_GUI_V3_1.py

Prepare SysCal executable, .PFC configuration, binary .TBL, receiver .CLB, and coil .CLC files. Optional .CLE files can be selected, but the SysCal invocation only passes a CLC wildcard; CLE use is not verified.

# 3. GUI fields

Input folder: source files to scan. Output folder: CTS destination, which must differ from the input folder.

SysCal EXE / PFC file / TBL file: paths to required inputs. CTS filename: a simple filename ending in .CTS. Timeout (s): maximum execution duration, default 120.

“Auto-detect files” scans the selected folder and receiver/coils subfolders. Ambiguous selections must be resolved manually.

# 4. Inspecting and editing TBL

Click “Load TBL”. Supported binary TBL files contain 25-byte records. The table displays record index, parameter name, and decoded value.

Double-click a value cell to edit a supported float64, int32, or text record. Complex binary byte records are read-only.

EXLN and EYLN represent electrode lengths (m); EGN/HGN and related fields are gain codes/settings and must not automatically be interpreted as physical gain multipliers.

Click “Find gain / spacing” to locate fields. “Save edited TBL as...” creates a separate file; the original TBL cannot be overwritten from the GUI.

# 5. Generate CTS

1. Choose the input folder and click “Auto-detect files”.

2. Confirm SysCal EXE, PFC, and TBL paths; load the TBL.

3. Select at least one receiver CLB and one coil CLC; optionally add other supported calibration files.

4. Review or edit electrode length and acquisition parameters as necessary.

5. Select an output folder and enter the CTS filename.

6. Click “Run SysCal → CTS” and read the execution log.

7. Confirm that a nonempty CTS was created at the requested destination.

# 6. Files, provenance, and safeguards

The GUI creates a timestamped working folder inside the output directory, copies executable/configuration/calibration files, and writes the edited TBL copy.

The working folder includes manifest.json (inputs, edits, command, and CLE caveat) and syscal_run.log (return code, standard output, and errors).

A CTS is accepted only when SysCal exits successfully and produces a nonempty output file. The GUI does not overwrite original input files.

# 7. CTS use in pyTRACMT

Use the CTS generated by the manufacturer’s SysCal to calibrate legacy Phoenix MTU-5A raw-count MTH5 in pyTRACMT. Confirm that the CTS belongs to the same receiver, sensors, gain/filter settings, and recording configuration.

Check the resulting impedance, apparent resistivity, and phase, and preserve the CTS and manifest for reproducibility.

# 8. Troubleshooting

No CTS: verify SysCal executable, PFC, TBL, CLB/CLC files, output permissions, and the execution log.

Unsupported TBL layout: this editor expects fixed 25-byte binary records.

Incorrect gain/spacing: inspect TBL values and manufacturer documentation; do not infer physical gain from a code alone.

Timeout or nonzero return code: inspect syscal_run.log and rerun only after correcting the underlying problem.