# pyTRACMT Professional v12
GUI Option Guide

## 1. Input Files

### Parameter File (param.dat)

The main control file used by TRACMT. It defines nearly all processing options, including DATA_FILES, SEGMENT, PROCEDURE, RRMS, MESTIMATORS, PREWHITENING, ROBUST_FILTER, COHERENCE_CRITERIA, and OUTPUT.

### Data Files

Time-series files for each channel, such as Ex, Ey, Hx, Hy, Hz, Hxr, and Hyr. These files can be specified directly in the GUI or loaded automatically from param.dat.

### Calibration Files

Sensor calibration files used to correct frequency response, amplitude, and phase. After the FFT, each spectrum is divided by the corresponding calibration response.

## 2. Procedure

### Ordinary RR (Procedure = 0)

Standard Remote Reference estimation using FFT → Cross Spectra → Ordinary Least Squares → Impedance estimation. This is the fastest processing method.

### Robust RRMS (Procedure = 1)

Uses the Fast-S estimator with the workflow: FFT → Candidate Subsets → Fast-S → IRLS → Tukey Weighting → Impedance. This is the most commonly used option in TRACMT.

### MRRMS (Procedure = 2)

Modified RRMS. This option is less commonly used.

## 3. M-estimators

None: No robust weighting; standard least-squares estimation.
Huber: Conservative robust weighting. Weight = 1 for |r|<k, and k/|r| for |r|>k.
Tukey Biweight: Default in TRACMT. Weight=(1-u²)²; observations beyond the cutoff receive zero weight.
Thomson: Alternative robust weighting method, used less frequently.

## 4. Huber Parameters

Threshold (e.g., 3) indicates that down-weighting starts at approximately 3σ. Iteration specifies the maximum number of robust iterations (e.g., 10).

## 5. RRMS Parameters

Typical values: 1, 100, 3, 0.05, 10, 16, 0.01

| Parameter | Description |
| --- | --- |
| Scale Estimation | Fast-S scale estimator. |
| Number of Subsets | Random subsets used by Fast-S. Larger values improve robustness but increase computation time. |
| Subset Size | Typically 3. |
| Convergence | Scale convergence criterion (e.g., 0.05). |
| Maximum Iterations | Maximum RRMS iterations (e.g., 10). |
| Candidate Subsets | Number of candidate subsets retained per iteration (e.g., 16). |
| Final Tolerance | Final convergence tolerance (e.g., 0.01). |

## 6. Prewhitening

None: No AR whitening.
Standard AR: Estimate an AR(p) model and filter the time series to reduce spectral leakage.
Robust AR: Robust PARCOR-like estimation after robust filtering.
Max AR Order: Maximum AR order (e.g., 30).
Candidate AR: Number of candidate AR models (e.g., 100).

## 7. Robust Filter

Enable the robust spike filter and specify Threshold 1, Threshold 2, and Maximum Consecutive Samples.
Threshold 1 (e.g., 10σ): Detect potential spikes.
Threshold 2 (e.g., 12σ): Immediately classify samples as spikes.
Maximum Consecutive Samples (e.g., 50): Prevent long sections from being entirely replaced.
Replace Data: Replace detected spikes with median-filtered values.

## 8. Coherence Criteria

Uses squared coherence (γ²). Segments with γ² below the selected threshold (e.g., 0.7) are rejected from the impedance estimation. This greatly improves MT data quality.

## 9. Bootstrap

Fixed-weight Bootstrap is the default TRACMT error estimation method. A typical value of 1000 performs 1000 bootstrap resamplings to estimate errors for impedance, apparent resistivity, and phase.

## 10. Rotation

0° indicates the standard North–East coordinate system. A value such as 30° rotates the impedance tensor by 30°.

## 11. Output

Response Functions: response_functions.csv containing Zxx, Zxy, Zyx, and Zyy.
Apparent Resistivity: Outputs apparent resistivity, phase, and associated errors.
EDI: Exports a standard *.edi file.

## 12. Output Level

Higher output levels (0, 1, 2) generate more detailed logs, including TRACMT.log and TRACMT.cvg.

## Recommended TRACMT-Compatible Settings

| GUI Option | Recommended Setting |
| --- | --- |
| Procedure | Robust RR (RRMS) |
| M-estimator | Tukey Biweight |
| RRMS Scale | 1 |
| Random Subsets | 100 |
| Subset Size | 3 |
| Candidate Subsets | 16 |
| Convergence | 0.01 |
| Prewhitening | Standard AR |
| Max AR Order | 30 |
| Robust Filter | ON |
| Threshold 1 | 10σ |
| Threshold 2 | 12σ |
| Coherence | ON |
| Coherence Threshold | 0.7 |
| Bootstrap | Fixed-weight Bootstrap |
| Bootstrap Number | 1000 |
| Rotation | 0° |