# pyTRACMT Professional v11

## Installation and Execution Guide

### 1. Overview

pyTRACMT Professional v11 is an open-source, Python-based magnetotelluric (MT) processing program designed to reproduce the principal processing framework and numerical procedures of TRACMT within a modern, platform-independent environment.

The program integrates the MT processing engine with a graphical user interface (GUI) and command-line interface (CLI). It supports time-series preprocessing, spectral analysis, remote-reference transfer-function estimation, robust regression, uncertainty estimation, calibration correction, apparent resistivity and phase calculation, visualization, and data export.

Version 11 is the TRACMT Source-Aligned Edition, with additional numerical alignment to the original TRACMT implementation. These improvements include TRACMT-style short-section zero padding, removal of unintended segment demeaning, C++-compatible std::mt19937_64 random-number generation for resampling procedures, source-aligned ordinary-RR coherence rejection, and TRACMT-compatible Tukey biweight scale calculations.

The implementation is intended to provide numerical compatibility with the TRACMT workflow and processing philosophy. Exact bit-for-bit identity may depend on the original TRACMT build, compiler and standard library, input precision, and preprocessing settings.

## 2. Main Functions

pyTRACMT Professional v11 supports:

- Ordinary Remote Reference (RR) estimation.

- Robust Multivariate Regression (RRMS).

- Modified RRMS (MRRMS) processing.

- Sequential M-estimators for robust ordinary RR processing.

- Fast-S-based robust multivariate estimation.

- Huber, Tukey biweight, and Thomson-type robust weighting.

- Parametric, bootstrap, jackknife, robust-bootstrap, and strict-bootstrap uncertainty estimation.

- Standard and robust prewhitening.

- Robust time-domain spike filtering.

- Squared-coherence-based segment rejection.

- High-pass, low-pass, and notch filtering.

- Frequency-domain calibration correction.

- Segment-based FFT spectral processing.

- Transfer-function and impedance estimation.

- Estimation of Zxx, Zxy, Zyx, and Zyy.

- Apparent resistivity and phase calculation.

- Uncertainty estimation for impedance, apparent resistivity, and phase.

- TRACMT-compatible processing and convergence logs.

- CSV output with intuitive MT component names.

- Basic EDI export.

- GUI-based parameter configuration and param.dat generation.

- Time-series data preview.

- Interactive response-function, apparent-resistivity, and phase plotting.

- Error-bar visualization.

- PNG figure export.

# 3. System Requirements

### Recommended System

- Windows 10 or Windows 11, 64-bit

- Python 3.10 or 3.11

- Minimum 8 GB RAM

- 16 GB RAM or more recommended for large datasets and bootstrap processing

- Anaconda or Miniconda recommended

The processing engine is written in Python and may also operate on other platforms supported by Python, NumPy, SciPy, and Tkinter.

# 4. Required Python Packages

### Core Packages

- numpy

- pandas

- scipy

- matplotlib

### GUI

- tkinter

Tkinter is normally included with standard Python distributions and is not usually installed through pip.

### Optional Packages

- openpyxl

- pyinstaller

openpyxl is useful for spreadsheet-related workflows, whereas pyinstaller is required only if a standalone Windows executable is to be created.

# 5. Installation Using Conda

### Step 1. Install Miniconda or Anaconda

Install a current Miniconda or Anaconda distribution.

### Step 2. Create a Dedicated Environment

Open Anaconda Prompt and run:

conda create -n pytracmt python=3.11 -y

### Step 3. Activate the Environment

conda activate pytracmt

### Step 4. Install the Required Packages

conda install -c conda-forge numpy pandas scipy matplotlib openpyxl -y

### Step 5. Test the Installation

python -c "import numpy, pandas, scipy, matplotlib; print('pyTRACMT environment OK')"

If the message

pyTRACMT environment OK

appears without an error, the core Python environment is ready.

### Step 6. Test Tkinter

Run:

python -c "import tkinter; print('Tkinter OK')"

If Tkinter OK is displayed, the GUI should be available.

# 6. Installation Using pip

Alternatively, a standard Python virtual environment can be used.

Create the environment:

python -m venv pytracmt_env

Activate it under Windows:

pytracmt_env\Scripts\activate

Install the required packages:

python -m pip install --upgrade pip
pip install numpy pandas scipy matplotlib openpyxl

Test the environment:

python -c "import numpy, pandas, scipy, matplotlib, tkinter; print('pyTRACMT environment OK')"

# 7. Running pyTRACMT

Place the pyTRACMT Python program in a working folder.

For example:

pyTRACMT_Professional_v11.py

Open Anaconda Prompt and activate the environment:

conda activate pytracmt

Move to the program folder:

cd C:\PyTRACMT_Project

Start the GUI:

python pyTRACMT_Professional_v11.py

If no command-line arguments are supplied, pyTRACMT opens the graphical user interface.

# 8. Command-Line Execution

pyTRACMT can also be executed directly from the command line using an existing param.dat file.

Basic syntax:

python pyTRACMT_Professional_v11.py param.dat

To specify a separate output folder:

python pyTRACMT_Professional_v11.py param.dat output_folder

For example:

python pyTRACMT_Professional_v11.py C:\MT_Data\param.dat C:\MT_Data\output

This mode is useful for automated processing or when GUI interaction is not required.

# 9. Graphical User Interface

The GUI is organized into a main window containing several tabs.

The principal tabs are:

### Parameter Builder

Provides graphical controls for creating the processing configuration, selecting input files, and generating param.dat.

### param.dat Content

Displays the current parameter-file contents and allows the configuration to be inspected or saved.

### Log

Displays processing messages and execution status.

### Response Functions

Displays the estimated transfer-function and impedance results in tabular form.

### ρa / Phase

Displays apparent resistivity, phase, and associated uncertainties.

### Plots

Provides graphical visualization of:

- Apparent resistivity

- Phase

- Response amplitude

- Response phase

The impedance component can be selected from:

- Zxx

- Zxy

- Zyx

- Zyy

Calculated uncertainty values are displayed as error bars when available.

Plots can also be exported as 300-dpi PNG files.

### Time-series Preview

Displays a preview of the input time-series data, including sample index and time when the sampling frequency is known.

# 10. Basic GUI Workflow

A typical GUI workflow is:

- Start pyTRACMT.

- Open the Parameter Builder tab.

- Specify the sampling frequency.

- Select the Ex, Ey, Hx, and Hy time-series files.

- Select remote-reference Hx and Hy files.

- Select the processing procedure.

- Configure robust estimators and uncertainty estimation.

- Configure segment lengths and frequency indices.

- Enable preprocessing options if required.

- Specify calibration files when sensor calibration is required.

- Generate param.dat.

- Review the generated configuration in the param.dat Content tab.

- Select the output directory.

- Run the calculation.

- Monitor processing in the Log tab.

- Inspect transfer functions in Response Functions.

- Inspect apparent resistivity and phase in ρa / Phase.

- Visualize results in Plots.

- Review input data in Time-series Preview.

- Export the resulting CSV, EDI, and figure files.

# 11. Input Data Requirements

The standard MT channel order is:

- Ex

- Ey

- Hx

- Hy

- Remote-reference Hx

- Remote-reference Hy

The current RR/RRMS workflow requires the two remote-reference magnetic channels.

The exact number of channels is determined from:

NUM_OUT + NUM_INPUT + NUM_RR

For standard MT processing, this normally corresponds to two electric outputs, two local magnetic inputs, and two remote-reference magnetic channels.

# 12. Supported Time-Series Files

pyTRACMT supports numeric time-series data stored in files such as:

- TXT

- DAT

- CSV

Files may contain headers and multiple columns.

When multiple numeric columns are present, the current reader uses the last valid numeric column as the channel data. Therefore, separate channel files are recommended unless the input format has been carefully checked.

The program also supports file paths containing spaces, including Windows paths.

# 13. Parameter File

pyTRACMT uses a TRACMT-style parameter file, conventionally named:

param.dat

The current implementation recognizes settings including:

NUM_OUT
NUM_RR
SAMPLING_FREQ
NUM_THREADS
NUM_SECTION
DATA_FILES
SEGMENT
OVERLAP
PROCEDURE
RRMS
MESTIMATORS
HUBER
THOMSON
TUKEYS_BIWEIGHTS
ERROR_ESTIMATION
BOOTSTRAP
PREWHITENING
ROBUST_FILTER
COHERENCE_CRITERIA
CAL_FILES
HIGH_PASS
LOW_PASS
NOTCH
NOTCH_PARAM_Q
ROTATION
AZIMUTH
START_TIMES
OUTPUT_RHOA_PHS
OUTPUT_EDI
OUTPUT_LEVEL

The GUI can generate the parameter file automatically, reducing the need for manual editing.

# 14. Processing Procedures

## PROCEDURE 0 — Ordinary Remote Reference

PROCEDURE
0

Performs ordinary remote-reference transfer-function estimation.

Robust M-estimation can optionally be applied through sequential M-estimators.

The available robust estimators include:

- None

- Huber

- Tukey biweight

- Thomson

This procedure provides the simplest starting point for verifying a dataset.

## PROCEDURE 1 — RRMS

PROCEDURE
1

Performs robust multivariate remote-reference regression using an RRMS-style framework.

The implementation includes Fast-S-based robust initialization and iterative robust estimation designed to follow the TRACMT processing methodology.

RRMS is generally preferable when the spectral data contain significant outliers or non-Gaussian noise.

## PROCEDURE 2 — Modified RRMS

PROCEDURE
2

Performs the modified multivariate regression workflow implemented in pyTRACMT.

This option provides an alternative robust multivariate processing strategy for comparison with standard RR and RRMS results.

# 15. M-Estimators

For ordinary RR processing, pyTRACMT supports sequential M-estimation.

The available choices include:

- None

- Huber

- Tukey biweight

- Thomson

A common robust sequence is:

Huber → Tukey

Huber weighting initially suppresses large residuals while retaining relatively high statistical efficiency, after which Tukey biweight provides stronger rejection of remaining outliers.

Version 11 contains the complete TRACMT Tukey biweight parameter table and follows the corresponding TRACMT interpolation and scale-update procedure.

# 16. Error and Uncertainty Estimation

The current implementation supports five uncertainty-estimation modes:

### Mode 0 — Parametric

Covariance-based parametric uncertainty estimation.

### Mode 1 — Fixed-Weights Bootstrap

Bootstrap resampling while retaining the robust weights obtained from the original solution.

This is useful for robust ordinary-RR processing when the estimator itself does not need to be refitted during every bootstrap realization.

### Mode 2 — Jackknife

Estimates uncertainty using jackknife resampling.

### Mode 3 — Robust Bootstrap

Performs bootstrap resampling and reruns the selected robust estimator for each realization.

### Mode 4 — Strict Bootstrap

Performs a complete refit of the selected processing procedure for every bootstrap realization.

Modes 3 and 4 are computationally more expensive but more fully represent uncertainty associated with the robust estimation process.

The program reports uncertainties for impedance and propagates them to apparent resistivity and phase where appropriate.

# 17. Spectral Processing

The time series are divided into overlapping segments according to the SEGMENT and OVERLAP settings.

Each segment is windowed using a periodic Hann window and transformed to the frequency domain using the FFT.

Version 11 follows important TRACMT source behavior:

- Short sections are zero-padded to the requested segment length.

- No additional per-segment demeaning is applied during spectral conversion.

- Frequency samples are selected using the specified FFT indices.

For a segment length (L), sampling frequency (f_s), and FFT index (k), the corresponding frequency is

f = k × fs / L

# 18. Coherence-Based Segment Rejection

The COHERENCE_CRITERIA option can be used to reject spectral segments with insufficient coherence.

Version 11 reproduces the ordinary-RR squared-coherence rejection definition used by the TRACMT workflow.

This can improve transfer-function estimation by removing poorly correlated or strongly contaminated spectral segments.

Coherence information and rejected-segment statistics are written to the convergence log.

# 19. Robust Filtering

The optional ROBUST_FILTER procedure is applied in the time domain before spectral analysis.

It is designed to identify anomalous spikes using robust statistics and replace selected contaminated samples with locally estimated values.

The filtering parameters control:

- Initial rejection threshold

- Strong-outlier threshold

- Maximum consecutive contaminated samples

Robust filtering should be used carefully because excessive filtering can modify genuine MT signals.

# 20. Prewhitening

pyTRACMT provides optional prewhitening before spectral estimation.

Available approaches include:

- No prewhitening

- Standard autoregressive prewhitening

- Robust AR/PARCOR-like prewhitening

The AR model is selected using statistical criteria, and the resulting coefficients are used to whiten the time series before FFT processing.

Prewhitening can reduce strong spectral coloration and improve statistical estimation under suitable conditions.

# 21. Digital Filtering

The current parameter structure also supports:

### High-Pass Filtering

HIGH_PASS

Suppresses frequencies below a specified cutoff.

### Low-Pass Filtering

LOW_PASS

Suppresses frequencies above a specified cutoff.

### Notch Filtering

NOTCH

Suppresses selected narrow-band frequencies.

The notch-filter quality factor can be controlled using:

NOTCH_PARAM_Q

These options can be useful for suppressing known instrumental or cultural noise before spectral processing.

# 22. Calibration Files

Frequency-domain calibration correction can be applied using:

CAL_FILES

A calibration file contains the sensor response as a function of frequency.

During processing, the Fourier-domain channel response is divided by the interpolated calibration response.

Before processing, confirm that:

- Calibration file paths are correct.

- The calibration frequency range covers the target MT frequency band.

- Calibration files correspond to the correct channels.

- Units are consistent with the selected input-data convention.

# 23. Units

The GUI includes unit modes for different input conventions, including practical MT and SI representations.

For TRACMT-compatible impedance expressed using electric fields in mV/km and magnetic fields in nT, apparent resistivity is calculated as

ρa = 0.2 T |Z|²

where:

- ρa is apparent resistivity in Ω·m,

- T is period in seconds,

- Z is the MT impedance in the corresponding practical units.

Phase is calculated from the complex impedance as

φ = atan2(Im(Z), Re(Z))

and reported in degrees.

Correct unit selection is essential because an incorrect impedance-unit convention can produce a large systematic error in apparent resistivity.

# 24. MT Impedance Components

For the standard two-output, two-input MT configuration, pyTRACMT estimates the complete impedance tensor:

Zxx
Zxy
Zyx
Zyy

The off-diagonal components

Zxy
Zyx

are commonly the principal components used for conventional MT interpretation.

However, Zxx and Zyy are retained because they contain useful information regarding three-dimensional structure, distortion, coordinate orientation, and data quality.

# 25. Apparent Resistivity and Phase

Apparent resistivity and phase are calculated for the available impedance components.

The principal output quantities include:

- Zxx apparent resistivity

- Zxx phase

- Zxy apparent resistivity

- Zxy phase

- Zyx apparent resistivity

- Zyx phase

- Zyy apparent resistivity

- Zyy phase

Corresponding uncertainty estimates are also included when available.

# 26. Output Files

A normal processing run may generate the following files.

### TRACMT.log

Main processing log containing parameter information, processing status, warnings, and execution information.

### TRACMT.cvg

Convergence and robust-processing log containing information such as robust iterations, coherence rejection, and estimator convergence.

### response_functions.csv

Estimated transfer functions and impedance components.

The current output uses intuitive component names such as:

Zxx
Zxy
Zyx
Zyy

rather than numerical index labels where possible.

### apparent_resistivity_and_phase.csv

Contains apparent resistivity, phase, and associated uncertainties for the MT impedance components.

### response_functions_gui_alias.csv

GUI-oriented version of the response-function results.

### apparent_resistivity_and_phase_gui_alias.csv

GUI-oriented apparent-resistivity and phase table with human-readable component labels.

### EDI Output

When enabled, pyTRACMT generates a basic EDI representation containing the available impedance tensor components:

ZXX
ZXY
ZYX
ZYY

including their real and imaginary parts.

# 27. Plotting Results

The Plots tab provides four principal plot types:

- Apparent resistivity

- Phase

- Response amplitude

- Response phase

The component selector allows:

- Zxx

- Zxy

- Zyx

- Zyy

When uncertainty information is available, the plotting system displays error bars.

The horizontal axis is generally represented using period for MT response visualization.

Plots can be saved as PNG files at 300 dpi.

# 28. Recommended First Test

For a new dataset, begin with a simple processing configuration before enabling the full robust workflow.

Recommended initial settings:

- Ordinary Remote Reference (RR)

- No prewhitening

- No robust filtering

- No coherence rejection initially

- Parametric error estimation or a small bootstrap test

- A limited number of frequency indices

- Correct Ex, Ey, Hx, Hy, RR Hx, and RR Hy channels

First verify:

- Zxy and Zyx are physically reasonable.

- Apparent resistivity curves are continuous.

- Phase values are physically plausible.

- Frequencies and periods are correct.

- Channel order and units are correct.

Robust processing options can then be enabled incrementally.

# 29. Recommended Robust Workflow

After the basic RR solution has been verified, a typical robust workflow is:

- Inspect the raw time series.

- Apply calibration correction if required.

- Apply robust filtering only if significant spikes are present.

- Enable prewhitening if appropriate.

- Apply coherence-based rejection.

- Run ordinary RR with Huber followed by Tukey.

- Compare with RRMS.

- Inspect Zxx, Zxy, Zyx, and Zyy.

- Inspect apparent resistivity and phase.

- Evaluate uncertainty estimates.

- Export the final CSV and EDI results.

This staged approach helps identify whether changes in the response are caused by preprocessing, robust regression, coherence rejection, or uncertainty estimation.

# 30. Performance Notes

Large datasets and robust resampling procedures can require substantial computation time.

The most computationally expensive options are generally RRMS and full-refit bootstrap procedures.

For testing:

- Use a shorter time-series interval.

- Use fewer frequency indices.

- Reduce the number of bootstrap repetitions.

- Begin with ordinary RR.

- Disable robust filtering and prewhitening initially.

- Enable robust options only after the basic response has been verified.

Final processing can then use more complete frequency coverage and uncertainty estimation.

# 31. Troubleshooting

### Error: ModuleNotFoundError: No module named ‘pandas’

Activate the pyTRACMT environment:

conda activate pytracmt

Install pandas:

conda install -c conda-forge pandas

### Error: ModuleNotFoundError: No module named ‘scipy’

Run:

conda activate pytracmt
conda install -c conda-forge scipy

### Error: Tkinter is not available

Test Tkinter using:

python -c "import tkinter"

If the import fails, install or reinstall a Python distribution that includes Tcl/Tk support.

The processing engine can still be executed through the command line when the GUI is unavailable.

### Error: DATA_FILES block is required

Check that param.dat contains a valid:

DATA_FILES

section and that the number of listed channels is consistent with the processing configuration.

### Error: SEGMENT block is required

The current program requires a valid:

SEGMENT

section defining segment length and FFT frequency indices.

### Error: Time-series file not found

Check:

- File path

- Filename

- Relative versus absolute path

- Current working directory

- Paths specified in param.dat

The current parser supports Windows paths containing spaces and performs additional path diagnostics when a file cannot be located.

### Error: No numeric samples read

Check that:

- The input file contains numeric data.

- The selected channel column contains valid numbers.

- The file is not empty.

- Header formatting is valid.

- The skip value does not exceed the available number of samples.

### Remote-reference warning

The currently implemented RR, RRMS, and MRRMS workflows require independent remote-reference magnetic channels.

Provide:

RR Hx
RR Hy

before starting the calculation.

### Calculation takes too long

Try:

- Fewer frequency indices

- Shorter input data

- Ordinary RR instead of RRMS

- Fewer bootstrap repetitions

- Parametric uncertainty estimation during testing

- Disabling full-refit bootstrap modes

After the processing configuration has been verified, restore the desired final settings.

# 32. Recommended Folder Structure

A convenient project structure is:

C:\PyTRACMT_Project
│
├── pyTRACMT_Professional_v11.py
├── param.dat
│
├── data
│   ├── Ex.csv
│   ├── Ey.csv
│   ├── Hx.csv
│   ├── Hy.csv
│   ├── Hxr.csv
│   └── Hyr.csv
│
├── calibration
│   ├── Ex.cal
│   ├── Ey.cal
│   ├── Hx.cal
│   ├── Hy.cal
│   ├── Hxr.cal
│   └── Hyr.cal
│
└── output
    ├── TRACMT.log
    ├── TRACMT.cvg
    ├── response_functions.csv
    ├── apparent_resistivity_and_phase.csv
    ├── response_functions_gui_alias.csv
    ├── apparent_resistivity_and_phase_gui_alias.csv
    └── pyTRACMT.edi

Calibration files are required only when calibration correction is used.

# 33. Suggested Workflow for New Data

For a new MT dataset:

- Confirm the sampling frequency.

- Confirm the channel order.

- Confirm the electric- and magnetic-field units.

- Inspect the raw time series.

- Verify remote-reference synchronization and data availability.

- Run a short ordinary-RR test.

- Inspect Zxy and Zyx.

- Inspect Zxx and Zyy for additional quality control.

- Check apparent resistivity and phase.

- Check uncertainty/error bars.

- Apply calibration correction where required.

- Test robust filtering if spikes are present.

- Test prewhitening if required.

- Apply coherence rejection.

- Compare ordinary RR with robust RR and RRMS.

- Select the preferred processing strategy.

- Perform final uncertainty estimation.

- Export CSV and EDI results.

- Save plots and processing logs for documentation.

# 34. Packaging as a Windows Executable

Install PyInstaller:

pip install pyinstaller

A single-file GUI executable can be created using:

pyinstaller --onefile --windowed pyTRACMT_Professional_v11.py

The executable will normally be created in:

dist\pyTRACMT_Professional_v11.exe

Scientific Python executables may be relatively large because Python, NumPy, SciPy, Matplotlib, and GUI dependencies must be packaged with the application.

If executable packaging causes dependency problems, running pyTRACMT from its dedicated Conda environment is recommended.

# 35. Important Notes on TRACMT Compatibility

pyTRACMT Professional v11 is a Python reconstruction of the principal TRACMT processing workflow rather than a direct compilation of the original C++ program.

Version 11 incorporates several source-aligned numerical procedures, including:

- C++ std::mt19937_64-compatible random-number generation

- TRACMT-style bootstrap resampling

- Full estimator refitting for robust and strict bootstrap modes

- TRACMT Tukey biweight parameter tables

- TRACMT-style Tukey interpolation and scale updating

- Short-section zero padding

- Source-aligned FFT segmentation behavior

- Removal of unintended segment demeaning

- TRACMT-style ordinary-RR coherence rejection

- TRACMT-compatible apparent-resistivity scaling

- TRACMT-style uncertainty propagation

These features are intended to improve consistency between pyTRACMT and the original TRACMT processing results.

Nevertheless, exact numerical identity cannot be guaranteed in all cases because results may also depend on compiler implementation, numerical libraries, floating-point precision, random-number distribution mapping, preprocessing settings, and input-data precision.

## End of Guide

pyTRACMT Professional v11 — TRACMT Source-Aligned Edition